Module 1: Real-Time and Embedded Systems

Task M1QP - Comprehensive Interrupt-Based Sense-Think-Act System

Summary and Reflection

Name : Harman

Student I.D: 2410994796

# Introduction:

The aim of building an interrupt-based system with an Arduino was to have sensors detect input; to have the microcontroller perform logic on that input; and to output a response without using delays or polling continuously. We used three digital sensors and two LEDs to accomplish this task. Our system is able to use various types of interrupts, allowing for fast reaction times and handling of multiple events at once.

# System Architecture and Logic

We used:

- Pir sensor on pin 8

- Tilt sensor on pin 9

- Slide switch on pin 2

- Event LED on pin 13

- Timer led on pin 12

Our system implements the Sense – Think – Act model.

## Sensing:

Sensors are used to detect motion, tilt, or position changes of the switch.

## Thinking:

The program checks to see if any flag variables are set in a main loop, if one is found to be set, then the program will determine what action to take.

## Acting:

LEDs will be turned ON and printed messages to the Serial Monitor.

Both the PIR and Tilt sensors will be attached to pin change interrupts because they are in the same pin group, so when one or the other changes state, then the interrupt will execute and set the corresponding flag.

The slide switch will be an attached interrupt with attachInterrupt().

The Timer1 interrupt was also used to create an event every second. As the timer counts to 1 second, the timer LED will change state, indicating that our system can simultaneously process both time-based and event-based tasks.

All interrupt functions are kept very short and basically will set the flags. The main loop will then do most of the work, which keeps our system safe and stable.

# Setup for Interrupts

In order to enable the Pin Change Interrupt and to configure the correct Pin Change Interrupt Control Register (PCICR) and Pin Change Interrupt Mask Register (PCMSK0) the following steps were taken. Pins 8 and 9 were configured to monitor both sensors.

The external interrupt was configured for pin 2 using the attachInterrupt() function to detect the CHANGE mode of the external interrupt. This allows for detection of HIGH and LOW transitions.

Timer1 was configured in CTC mode (Clear on Timer Compare) with a prescaler set at 1024; thus the OCR1A register was set so that a time interrupt occurs once per second based on a 16 MHz clock.

To ensure that any variables that are modified within the ISR and then referenced from the main loop were declared as volatile.

The code does not contain the delay() function.

# Difficulties Faced

One of the difficulties was initially setting up a Pin Change Interrupt properly - this received no response previously because the correct pin mask had to be selected. Following careful checking of the pin group, this became successful.

In the initial stages of development, I encountered minor logic problems that resulted in the LED not responding in accordance with expected results; I resolved these through testing various conditions for each sensor.

Once these smaller issues were addressed, the system was able to perform successfully and multiswitch interrupts successfully.

# Conclusion

Through this project we were able to learn about the interrupt based systems used in embedded programming, we learnt to implement the Pin Change Interrupt, External Interrupt and Timer Interrupt all within the same system.

The final system was able to handle both sensor and timer events simultaneously in a stable and efficient manner.